OTA

- First, use the Bluetooth app to find the target device, and record the mac address of the device.

- Download the upgrade file from the specified location and save it to your cell phone

- Moving files to the root directory of the mobile file management system

- After entering the upgrade app program, follow the previously recorded mac address to find the target device

- Select the device and click enter, then select the upgrade file

- Click on the device to enter the upgrade page ,If need a key, it's usually 0000000002.

- Note:If it fails, the name of the current device changes to PPlusOTA, and the mac address is incremented by 1.

- Repeat the upgrade steps, but this time do not enter your password!

- Wait 12 seconds for the initialization and then the progress bar will start to change.

- Use the Bluetooth app to scan the target device and verify that the upgrade was successful.